# Pass 6-24 Three Way Crossover Dossier

Stereo active crossover implementation and wiring record

This dossier records the accepted three-way implementation of the Pass 6-24 active crossover. It is intended to be the practical reference for assembly, power-up and signal tracing. The design uses two complete boards, one for each stereo channel, with selected sections omitted or repurposed to obtain low, mid and high outputs.

# Current status

| Gate | Status | Resume point |
| --- | --- | --- |
| Architecture | Accepted | Two boards form a stereo three-way system. |
| Crossover frequencies | Accepted target | 300 Hz between low and mid; 1.9 kHz between mid and high. |
| Component population | In progress | Populate and inspect both boards against the final section map. |
| Inter-board wiring | Open | Complete L High to R Low band-pass link and external outputs. |
| Power-up | Open | Verify the single 24 V rail, current draw and stage DC conditions. |
| Signal verification | Open | Use the oscilloscope to trace low, mid and high outputs at known frequencies. |

# System role

The crossover sits after the DAC or line-level front end and before the three amplifier bands.

LOW feeds the 12 inch LM3886 bass monoblocks.

MID feeds the 10 inch Bianco midrange amplifier.

HIGH feeds the amplifier assigned to the CD44-PK compression drivers.

The crossover is active; there are no passive speaker-level crossover components in the three-way signal path.

# Accepted topology

| Board / section | Function | Implementation |
| --- | --- | --- |
| Board L Low | Left low-pass | 300 Hz low-pass output. |
| Board L High | Intermediate filter section | 300 Hz high-pass section; output stage partly omitted. |
| Board R Low | Intermediate filter section | 1.9 kHz low-pass section; input section partly omitted and tapped at the T4 bias resistor. |
| Board R High | High-pass | 1.9 kHz high-pass output for the high-frequency amplifier. |

The L High signal is linked into the R Low section to form the mid band-pass path. This is the central implementation decision: the two boards are not simply two independent two-way crossovers.

# Final section map

| Output | Path | Band |
| --- | --- | --- |
| LOW | L Low output | Below approximately 300 Hz |
| MID | L High through the modified R Low section | Approximately 300 Hz to 1.9 kHz |
| HIGH | R High output | Above approximately 1.9 kHz |

# Intentional omissions and modifications

L High output stage: omit the 100 ohm resistor, 47 kohm resistor and 10 uF output capacitor associated with the normal output stage.

R Low input section: omit the normal input section and inject the signal at the base of the T4 bias resistor as established in the accepted wiring plan.

Gain adjustment: use one 50 kohm trimmer at the L High input, before the modified filter path. There is no trimmer on the R Low input.

The supplied kit JFETs are matched JCJ113N21 devices and remain assigned to this crossover design.

# Crossover targets

| Parameter | Target / accepted value |
| --- | --- |
| Low to mid crossover | 300 Hz |
| Mid to high crossover | 1.9 kHz |
| Filter slopes | Use the populated Pass 6-24 section values; final measured slope to be confirmed. |
| Supply | Single 24 V DC rail |
| Gain trim | 50 kohm at L High input only |
| Signal reference | Line-level DAC / preamplifier output |

# Assembly and test sequence

Identify the left and right boards and mark them permanently before population.

Populate the normal components first, then verify every intentional omission against this dossier and the schematic.

Inspect the JFET orientation and matched-device allocation before applying power.

Power each board separately from a current-limited 24 V supply and record current draw.

Verify the 24 V rail at the board and inspect for abnormal heating or oscillation.

Inject a 1 kHz sine wave and confirm signal continuity through the normal low and high paths.

Test the modified mid path at 100 Hz, 300 Hz, 1 kHz, 1.9 kHz and 10 kHz.

Use the new oscilloscope to verify attenuation, phase behaviour and clipping at LOW, MID and HIGH outputs.

Set the 50 kohm input trimmer only after the three bands are electrically verified.

Record the final output levels used to drive the 12 inch, 10 inch and CD amplifiers.

# Known risks

The band-pass construction depends on the inter-board L High to R Low link; a wiring mistake can remove or corrupt the mid band.

The modified R Low input is not a standard populated input and must be checked against the accepted tap point.

The 50 kohm gain trimmer must not be duplicated at the R Low input.

Measured crossover frequency, slope, phase and output level remain open until the completed stereo unit is tested.

Do not connect the crossover to the final amplifier stack until the outputs have been checked with a dummy load or safe input instrument.

# Where to resume

Start with the physical board identification and the final population checklist. Then power each board independently at 24 V, document current draw and verify the normal low/high paths before wiring the modified mid band. The first useful scope session is a five-frequency sweep through all three outputs.

# Source and confidence note

This dossier records the accepted wiring decisions from the RIOT project work through 15 October 2026. Values and intentional omissions are treated as accepted design decisions; measured response and final assembly state remain open until the completed boards are tested.