# RIOT Electric Ballroom Project Index

Engineering archive and resume map

This index is the entry point for the RIOT home-audio project. It records the current state of each subsystem, separates verified evidence from engineering targets, and identifies the first action required when work resumes.

# Archive rule

The project dossier is the source of truth for accepted decisions and measured results. Chat discussions remain useful for exploration, but a decision is not considered closed until it is recorded in the relevant dossier.

# Current project map

| Project | Status | Resume point |
| --- | --- | --- |
| 12 inch bass system | As-built and operating | Sealed LF cabinets; approximately 40-300 Hz; LM3886 monoblocks; room integration and final measurements remain open. |
| 10 inch midrange system | Built and integrated; cabinet completion ongoing | SB Audience Bianco 10MW150; approximately 300 Hz to 1.8-2.2 kHz; waveguide/cabinet geometry documented; final cabinet and measurement work remain. |
| CD44-PK horn system | Designed and in use | Compression driver on horn; approximately 1.9 kHz upward; horn geometry and printing options documented; final acoustic verification remains. |
| Pass 6-24 three-way crossover | Assembly priority | Two boards used as one stereo three-way system; 300 Hz and 1.9 kHz sections; wiring and power-up verification remain. |
| LM3886 bass monoblocks | Completed and operating | Gainclone-style monoblocks with CRC supplies; assigned to the 12 inch bass section. |
| Pass Aleph Mini | Assembly priority | Vendor PCB; IRFP240 and IRFP9610 stages; approximately 8 W Class A target; matching, assembly, bias and commissioning remain. |
| RIOT Headbanger EF2 | Routed; engineering review complete; not released | Rev A mono PCB and validation package supplied. Real-device bring-up, thermal, protection and load testing remain. |
| LSK170/U310 CCS cascode | Designed; scope validation pending | JFET front end with measured device-specific operating points; large-signal compression and headroom investigation awaits the oscilloscope. |
| Ciuffoli power follower | Future build | Tube front end and MOSFET follower concept; defer until Aleph Mini, 6-24 and Headbanger gates are closed. |
| Digital audio chain | Operating configuration | Debian cameraHTPC, Amanero DAC, MPD, PipeWire, CamillaDSP and Firefox/Qobuz routing documented. |
| Room acoustics | Measured; treatment planned | Room II geometry, REW measurements, SBIR nulls, speaker positions and ceiling-cloud/front-wall treatment plan recorded. |
| Bench power supply | Design and procurement decision stage | Dual rails, 0 V reference, current limiting and safe repeatable bench operation required for upcoming commissioning work. |
| Measurement bench | Scope ordered; commissioning pending delivery | HANMATEK DOS1102S with integrated generator; initial validation will cover channels, probe compensation and generator output. |

# Recommended work sequence

| Order | Project | Closure gate |
| --- | --- | --- |
| 1 | Pass Aleph Mini | Finish construction, power supply, heatsinking and controlled bias commissioning. |
| 2 | Pass 6-24 three-way | Complete both channels, verify rails and signal routing, then document the final wiring. |
| 3 | RIOT Headbanger EF2 | Keep Rev A frozen; complete the staged prototype tests before fabrication release. |
| 4 | LSK170/U310 cascode | Use the new oscilloscope to measure gain, headroom, clipping and compression. |
| 5 | Ciuffoli power follower | Start only after the preceding gates are closed and documented. |

# Documentation status

The Headbanger Rev A engineering review is the first complete evidence-based dossier. The remaining dossiers will be populated from their actual schematics, drawings, measurements and photographs where available; recalled project history will be labelled as such until source evidence is added.

# Session handover rule

Record what physically changed.

Record what was measured or verified.

Record what remains uncertain.

Write the first exact action for the next session.