# RIOT Loudspeaker Systems Dossier

12 inch bass, 10 inch Bianco midrange and CD44-PK horns

This dossier records the physical loudspeaker systems and their role in the active three-way RIOT architecture. Room measurements remain primarily in the Room Acoustics dossier and are linked here by speaker and measurement metadata.

# System architecture

| Band | Transducer | Current target | Amplifier assignment |
| --- | --- | --- | --- |
| LOW | 12 inch bass system | Below approximately 300 Hz | LM3886 bass monoblocks; Pioneer SA-8100 currently used during integration |
| MID | SB Audience Bianco 10MW150 | Approximately 300 Hz to 1.8-1.9 kHz | LM3886 Gainclone monoblocks currently; Ciuffoli considered/planned |
| HIGH | SB Audience CD44-PK on custom horn | Above approximately 1.9 kHz | Aiyima A70 currently; Pass Aleph Mini planned |

# 12 inch bass cabinets

The current archive describes the cabinets as bass-reflex/Helmholtz systems, not transmission lines.

Earlier design record: approximately 72 L chamber with two front slots, each approximately 444 x 25 x 180 mm; total vent area approximately 222 cm2.

Earlier tuning estimates range approximately 48-53 Hz depending on the design revision.

The intended behaviour is a low-stress, high-headroom kick/bass engine rather than a small woofer driven near its limits.

Cabinet photographs, exact as-built dimensions and final tuning measurement remain to be attached.

# 10 inch Bianco cabinets

Driver: SB Audience Bianco 10MW150.

Target role: live-scale body and violence through the midrange, approximately 300 Hz to 1.8-1.9 kHz.

Recorded geometry for the waveguide-in-cabinet trial: mouth approximately 480 mm, throat-side geometry approximately 210 mm, depth approximately 212 mm, R500 flare.

Rear chamber record: approximately 150 x 442 x 290 mm; coil-to-back clearance approximately 87 mm.

Cabinet completion and final acoustic measurement remain open.

# CD44-PK horns

Compression driver: SB Audience CD44-PK, 1 inch CD.

Custom horn is intended for approximately 1.9 kHz upward.

IWATA-600 and IWATA-300 references were acquired; an oblate-spheroid/OS waveguide alternative was also considered.

Horn depth is approximately 220 mm in the system record.

Home 3D printing is part of the construction plan; final printed geometry and acoustic verification remain open.

# Listening evidence

The project target is live real-fi: physical impact, immediate attack, believable scale and effortless dynamics rather than audiophile politeness.

Recorded listening reports include strong snare attack without added decay, compelling tribal-percussion timing and bodily kicks that do not obscure the drum solo.

Earlier listening references include Private Investigations, Time, Angel, Chocolate Chip Trip, Keith Don’t Go and Cosmic Girl.

# Where to resume

Finish the physical 10 inch cabinets and horn printing, then run the fixed-position REW set from the Room Acoustics dossier. Do not interpret a speaker response without recording the amplifier, crossover and source chain used for that capture.

# Evidence status

System bands and current amplifier assignments are explicit project facts. Some cabinet dimensions, port dimensions, tuning values and driver data belong to different design revisions; the final as-built measurements must be tagged by revision before being treated as authoritative.